Department of Computer Science and Engineering

Course Title: Internet Of Things

Code: CSE406

Section: 1

LAB-02

Submitted To:

Dr. Raihan Ul Islam

Associate Professor

Department of Computer Science & Engineering

Submitted by

Name: Shairin Akter Hashi

ID: 2022-2-60-102

TASK:  How Water Level Sensor Works and Interface it with Arduino

Introduction

In IoT applications, real-time monitoring of environmental parameters is critical. This lab explores water level detection using an analogue sensor interfaced with an Arduino UNO, providing both serial feedback and LED-based status indicators. The objective is to design and implement a system that accurately measures water level and categorizes it as Empty, Low, Medium, or High.

Tools and Devices Used

- Arduino UNO.

- Analog Water Level Sensor.

- LEDs (Red, Yellow, Green).

- Resistors.

- Breadboard and Jumper Wires.

- Arduino IDE (v1.8+).

- USB Cable for Arduino.

- Serial Monitor (via Arduino IDE).

Process

1. Sensor Setup:

The water level sensor was powered through pin 7 (controlled digitally), and its analogue output was read from pin A0. A short delay was used to stabilize readings.

2. Threshold Definition:

Two thresholds were set:

- Lower threshold: 200

- Upper threshold: 400

3. LED Logic Control:

Based on the sensor value:

- Red LED indicates high water level

- Yellow LED indicates medium level

- Green LED indicates low level

- All LEDs are off when the sensor reads empty

4. Data Feedback:   Sensor readings and water level status were printed to the serial monitor every second for live tracking.

Arduino UNO Code:

// Sensor pins

#define sensorPower 7

#define sensorPin A0

// Value for storing water level

int val = 0;

int lowerThreshold = 200;

int upperThreshold = 400;

// Declaring pins to which LEDs are connected

int redLED = 2;

int yellowLED = 3;

int greenLED = 4;

void setup() {

Serial.begin(9600);

pinMode(sensorPower, OUTPUT);

digitalWrite(sensorPower, LOW);

// Set LED pins as an OUTPUT

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

// Initially turn off all LEDs

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

}

void loop() {

int level = readSensor();

if (level == 0) {

Serial.println("Water Level: Empty");

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

} else if (level > 0 && level <= lowerThreshold) {

Serial.println("Water Level: Low");

digitalWrite(redLED, HIGH);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

} else if (level > lowerThreshold && level <= upperThreshold) {

Serial.println("Water Level: Medium");

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, HIGH);

digitalWrite(greenLED, LOW);

} else if (level > upperThreshold) {

Serial.println("Water Level: High");

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, HIGH);

}

delay(1000);

}

//This is a function used to get the reading

int readSensor() {

digitalWrite(sensorPower, HIGH);

delay(10);

val = analogRead(sensorPin);

digitalWrite(sensorPower, LOW);

return val;

}

Output Image:

Image:1

Image:2

Image:3

Image:4

Result and Discussion:

The circuit successfully categorized water levels using the predefined thresholds. LED indicators responded accurately:

- No sensor detection resulted in all LEDs off and an “Empty” status.

- Readings above 400 activated the red LED, confirming high water presence.

- Values between 200 and 400 lit the yellow LED, indicating a medium level.

- Values below 200 triggered only the green LED, signalling low water.

Conclusion:

In this experiment, we successfully developed and tested an Arduino-based water level monitoring system using an analogue sensor and a simple LED-based visual feedback mechanism. The system effectively measured water levels and triggered corresponding LED indicators based on predefined thresholds, providing real-time status updates via serial communication.

GitHub Link:  https://github.com/Shairin207/IoT-Lab-Assignments